Mathematical Methods For Science and Engineering

Lecture Note 4

Nonlinear System of Equations

4.1  Newton-Raphson Method

Recall that the Newton-Raphson formula to find the root of the equation fx=0 can be written as

xn+1=xn-f'xn-1 fxn.

For a system of nonlinear equations, the Newton-Raphson formula in matrix form can be expressed as

Xn+1=Xn-JXn-1 FXn.

where X is a variables matrix, F(X) is a function matrix and J(X) is a Jacobian matrix.

The variable, function and Jacobian matrices for a system of two variables can be written as

X=xy,       F=f1(x,y)f2(x,y),         Jx,y=∂f1∂x∂f1∂y∂f2∂x∂f2∂y.

This method can be extended for equations with more variables.

Example 4.1  Consider the nonlinear systemx2+y-1&=0y3-2x+4=0 .

- Find the Jacobian matrix J(x,y) for the above system.

- Evaluate the inverse of the Jacobian matrix at (1.4, -1).

- Write down the iterative formula for the above system based on the Newton-Raphson method.

- Estimate the root to 2 d.p. using the using the above iterative formula once starting with x0, y0=(1.4, -1).

- Write down MATLAB commands to execute the iteration four times.

- Use MATLAB function “fsolve(fun,x0)” to find the above root.

Solution:

(a)  The function matrix and Jacobian matrix are

Fx,y=x2+y-1y3-2x+4,                Jx,y=2x1-23y2

(b)

Note: The inverse of a 2×2 matrix A=abcd can be calculated by

A-1=1Ad-b-ca

At the point  (1.4, -1). we have

J1.4,-1=2.81-23

and

J1.4,-1-1=0.2885-0.09620.1923   0.2692

(c)  The iterative formula is

Xn+1=Xn-JXn-1 FXn.

where

Xn=xnyn.

(d) Using n=0, we  have

X1=X0-JX0-1F(X0)

where

X0=1.4-1,     FX0=F1.4,-1=-0.040.2,

and

JX0-1=J1.4,-1-1=0.2885-0.09620.1923   0.2692

Thus

x1y1=1.4-1-0.2885-0.09620.1923   0.2692-0.040.2=1.4308-1.0462

root  to 2 d.p. is

x1=1.43    and   y1=-1.05.

Further iterations can be done by updating X0 by X1.

(e) Define the functions as        F=x2+y-1y3-2x+4

we have the Jacobian                   J=2x           1-2             3y2.

>> clear

>> F=@(x,y) [x.^2+y-1; y.^3-2*x+4];

>> J=@(x,y)[2*x,  1;  -2,  3*y.^2];

>> x(1)=1.4;

>> y(1)=-1;

>> for n=1:50

Xn=[x(n); y(n)];

Fn=F(x(n), y(n));

Jn=J(x(n), y(n));

Xn1=Xn-Jn\Fn;

x(n+1)=Xn1(1);

y(n+1)=Xn1(2);

end

>> Roots=[x',y']

Roots =

1.4000   -1.0000

1.4308   -1.0462

1.4299   -1.0447

1.4299   -1.0447

1.4299   -1.0447

(f)  >> clear

% Left_hand side of equations as vector

>> F=@(x) [x(1)^2+x(2)-1; x(2)^3-2*x(1)+4];

% Guess solution

>> x0=[1; -1];

% Solve the system

>> x=fsolve(F, x0)

x =

1.4299

-1.0447

4.2  Fixed Point Iteration Method for Nonlinear System

Consider the nonlinear system of two variables of the form

f1x,y=0     andf2x,y=0                                                     (1)

By rearrangements the system can be expressed as

x=g1x,yand     y=g2x,y                                                 (2)

If there is a point (p, q) such that

p=g1p,qand    q=g2p,q

then the point (p, q) is a fixed point of the system and it is a root of the system.

With the arrangement (2), we may assume an iterative formula

xn+1=g1xn, yn and yn+1=g2xn, yn                                    3

wheren=0, 1, 2, 3,∙ ∙ ∙  .

Without further details it may be mentioned that if x0, y0 is close to the fixed point (p, q) and if

∂∂xg1(x0,y0)+∂∂yg1(x0,y0)<1                                                      (4a)

and

∂∂xg2(x0,y0)+∂∂yg2(x0,y0)<1                                                      (4b)

then the iterative formula (3) will converge to the fixed point (p, q).

If the conditions (4) are not satisfied, the iterative process might diverge.

The above method can be extended for more than two variables.

Seidel Iteration

An improvement of the iteration process is the Seidel iteration similar to Gauss-Seidel iteration for linear system. In Seidel iteration process the iterative formulas are

xn+1=g1(xn,yn)

yn+1=g2(xn+1, yn)

Example 4.2

Consider the system of equations	y=x2-5x+3

x2+4y2=4

- Plot the graphs of the above system using MATLAB function “ezplot(fun)”.

From your plot find the number of real roots and estimate the roots.

- A fixed point iteration formula is suggested  to estimate root at  (x0, y0)

xn+1=15xn2-yn+3

yn+1=1104-xn2-4yn2+10yn

- Verify whether the above iterative formula will converge to the root near x0, y0=(0.4, 1.0). If converges, perform one iteration otherwise suggest another fixed point iterative formula which converge to root.

- Verify whether the above iterative formula will converge to the root near x0, y0=(0.9, -1). If converges, perform one iteration otherwise suggest another fixed point iterative formula which converge to the root.

- Write MATLAB commands to execute the iterative formula in b(ii) five times.

(a)  MATAB codes for ezplot

>> clear

>> figure

>> hold on

>> ezplot('y-x.^2+5*x-3')

>> ezplot('x.^2+4*y.^2-4')

>> hold off

From graph it can be seen that the system has two real roots near (0.4, 1.0)and (0.9, -0.9).

(b) (i) From the iterative formula let us define

g1(x,y)=15x2-y+3

and

g2(x,y)=1104-x2-4y2+10y

Here

∂∂xg1x,y=2x5 ,         ∂∂yg1x,y=-15

and

∂∂xg2x,y=-2x10 ,         ∂∂yg2x,y=110(-8y+10)

Near (0.4, 1.), we have

∂g1∂x+∂g1∂y=0.16+-0.2=0.36<1

and

∂g2∂x+∂g2∂y=-0.08+-81+1010=0.28<1

The partial derivative tests are satisfied and hence the iterations converges to the root near (0.4, 1.0).

We can use iterative formulas for the root near (0.4, 1.0) as

xn+1=15(xn2-yn+3)

yn+1=1104-xn2-4yn2+10yn

In Seidel iteration the second iterative equation will be of the form

yn+1=1104-xn+12-4yn2+10yn

Calculations with the above iterative formulas with x0=0.4 and  y0=1.0 are given below:

| Jacobi iteration |  |  |  | Seidel iteration |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| n | xn | yn |  | n | xn | yn |
| 0 | 0.4 | 1.0 |  | 0 | 0.4 | 1.0 |
| 1 | 0.432 | 0.984 |  | 1 | 0.432 | 0.9813 |

(ii) Near (0.9, -1.0) we have

∂g1∂x+∂g1∂y=0.36+-0.2=0.56<1

g1x,y) satisfy the partial derivative test.

But                        ∂g2∂x+∂g2∂y=-0.18+-8-1+1010=1.98>1

Test fails and the convergence is not guaranteed.

Rearranging the second equation by

x2+4y2-4+10y=10y

we have

y=110x2+4y2-4+10y .

Corresponding iterative function can be of the form

g3(x,y)=110x2+4y2-4+10y .

and

∂∂xg3x,y=2x10 ,         ∂∂yg3x,y=110(8y+10)

At (0.9, -1),

∂g3∂x+∂g3∂y=1.810+8-1+1010=0.38<1

g1x,y) satisfies the partial derivative test.

Thus a fixed point iterative formula which converge to root near (0.9, -1) is

xn+1=15(xn2-yn+3)

yn+1=110xn2+4yn2-4+10yn

(e) >> clear

>> x(1)=0.9;

>> y(1)=-1;

>> for n=1:5

x(n+1)=(x(n)^2-y(n)+3)/5;

y(n+1)=(x(n)^2+4*y(n)^2-4+10*y(n))/10;

end

>> Iterative_Roots =[x',y']

Iterative_Roots =

0.9000   -1.0000

0.9620   -0.9190

0.9689   -0.8886

0.9655   -0.8789

0.9622   -0.8767

0.9605   -0.8767

Exercise 4

1. Consider the following nonlinear systems:

(a)  x3+y2=2, x2-y=1 starting with x0, y0=(1.2, -0.5)

(b)  3x2-y2=0,    3xy2-x3-1=0   starting with x0, y0=(1, 1).

(c) lnx2+y2+y=1, x+xy2=0   starting with     x0, y0=(2.4, -0.6).

(d)  4x2-20x+14y2+8=0,12xy2+2x-5y+8=0   starting with x0, y0=(0.5, 1.9).

(e)  ysinxy+2=0,   cosxy+x-y=0   starting with x0, y0=(1, 2).

(f) tan-1x+ey2=5,cosx2+xy=10      starting with x0, y0=(-1.6, -1.3).

Now using Newton-Raphson method answer the following questions. For each of the above nonlinear system of equations:

(i) Find the Jacobian matrix  J(x,y) for the above system.

(ii)Evaluate the inverse of the Jacobian matrix at the given point.

(iii) Write down the iterative formula for Newton-Raphson    method.

(iv) Estimate the root to 2 d.p. using the using the above iterative formula once starting with the given point.

(v)Write down MATLAB commands to execute the iteration four times.

(vi) Use MATLAB function “fsolve (fun,x0)” to find the root.

.

2. Sketch the curves represented by the following non-linear equations,

2ex+y=0,

3x2+4y2=8.

- Solve using Newton’s method with initial estimate -1,-2.

- Repeat (a) but with an initial estimate -2,0.

- Use graphical approach to validate the results in (a) and (b).

3. Sketch the curves represented by the following non-linear equations,

x2-y2=4,

x2+y2=16.

- Solve using Newton’s method with initial estimate 3.0,2.5.

- Repeat (a) but with an initial estimate -3,-2.5.

- Use graphical approach to validate the results in (a) and (b).

4. The system of non-linear equations 4x+lnx2+y=1 and 8y-lnx+y2=1 has a solution near x0, y0=(1, 2). The following iterative formula is suggested to estimate the roots. xn+1=14(1-ln(xn2+yn)) yn+1=18(1+ln (xn+yn2))

i. Verify whether the above iterative formula will converge to the root near x0, y0=(1, 2). If converges, perform one iteration otherwise write ‘Not Convergent’.

ii. Write MATLAB commands to execute the iterative formula five times.

.

5. Determine the roots of the following simultaneous non linear equations using (a) Newton-Raphson method, (b) fixed-point iteration method.

x=y+x2-0.5,

y=x2-5xy

Employ initial guesses of x0=y0=1.0 and discuss the results.

6.	The following system has a root near x0, y0=(0.2,  0.3).

2x2+y2-15x+2=0,

xy2+x-10y+5=0.

Estimate the root correct to 3 decimal places using fixed point iterative method.

7. A planar, two-link robot arm is shown in the figure. The coordinate system xy is the tool frame and attached to the end-effector. The coordinates of the end-effector relative to the base frame are expressed as

x=L1cosθ1+L2cosθ1+θ2

y=L1sinθ1+L2sinθ1+θ2

Suppose the lengths, in consistent physical units, of the two links are L1=1 and L2=2, and that x=2.5, y=1.4. Find the joint angles θ1 and θ2 (in radians) using Newton’s method with initial estimate of (0.8,0.9)